Game Boy Emulator on STM32F411
PPU Performance Investigation Report

Status: Profiling / bottleneck isolation
15 September 2026

# 1. Purpose

This report records the experimentally confirmed performance findings in the STM32F411 Game Boy emulator PPU investigation, separates confirmed facts from hypotheses, and lists what remains unexplained. It is intended as a checkpoint for continuing optimization from measurements rather than guesses.

# 2. Current context

Target MCU: STM32F411CEU6.

Measured MCU clock: 60 MHz.

Game Boy core is C++ and is compiled for STM32 with arm-none-eabi-g++.

The existing Game Boy framebuffer is used directly; no second framebuffer/copy is desired.

USB CDC is used for diagnostic output.

STM32 DWT CYCCNT is the timing source.

STM32-specific timing is supplied to the portable PPU through a callback; ppu.cc should not directly depend on DWT.

# 3. Measurement method

DWT CYCCNT is enabled on the STM32. The PPU exposes accumulated counters for Background FIFO, Sprite FIFO, Sprite Scan, and Framebuffer. A platform-independent callback supplies the current cycle count to ppu.cc. Total frame time is measured around GameBoy_RunFrame(). At 60 MHz, milliseconds are calculated as cycles / 60000.

# 4. Confirmed profiling result

| Region | Cycles | Approx. time @ 60 MHz | Status |
| --- | --- | --- | --- |
| Total frame | 77,344,804 | 1,289 ms | Confirmed |
| BG FIFO | 5,300,566 | 88.3 ms | Confirmed |
| Sprite FIFO | 0 | 0 ms | Confirmed in tested frame |
| Sprite Scan | 20,041,431 | 334.0 ms | Confirmed |
| Framebuffer | 11,227,067 | 187.1 ms | Confirmed |
| ppu_drawing_mode() calls | 24,198 | — | Confirmed |

# 5. What is confirmed

## 5.1 PPU is the dominant performance problem

Earlier controlled measurements showed approximately 1,194 ms/frame with the PPU enabled and approximately 173 ms/frame with the PPU disabled. This establishes that PPU execution accounts for the overwhelming majority of the emulator's frame time.

## 5.2 Sprite Scan is a major bottleneck

Sprite Scan measured about 20.0 million cycles, approximately 334–335 ms/frame. This is a large fraction of the total 77.3 million cycles.

## 5.3 Sprite checking executes repeatedly

One frame produced 24,198 calls to ppu_drawing_mode(). The sprite-check section therefore executes many times per frame. With a maximum of ten loop iterations per call, the loop can represent up to about 241,980 iterations per frame.

## 5.4 Removing the sprite-check loop gives a large speedup

When the entire sprite-check loop was temporarily removed, total time changed from approximately 1,285 ms/frame to approximately 940 ms/frame, while Sprite Scan became 0 cycles. The improvement was approximately 345 ms/frame, closely matching the separately measured Sprite Scan cost. This is strong A/B evidence that the sprite-check loop is responsible for the measured Sprite Scan cost and materially affects total performance.

## 5.5 get_sp_byte() alone is not proven to be the cause

The sprite-X access through get_sp_byte() was replaced by a dummy 'u8 sprite_x = 0'. Sprite Scan remained about 20.32 million cycles instead of about 20.04 million cycles. The run-to-run difference is small compared with the measured cost. Therefore get_sp_byte() alone is not supported as the primary explanation.

# 6. What remains unexplained

## Exact expensive operation inside Sprite Scan

The sprite-check loop is confirmed expensive, but the individual statement(s) responsible for the ~20 million cycles have not yet been isolated.

## Why the apparently small loop costs so much

The loop has only up to ten iterations per ppu_drawing_mode() call, yet the accumulated cost is very large. The reason for the high per-call/per-iteration cost remains unknown.

## The remaining total-frame cost

The measured BG FIFO + Sprite Scan + Framebuffer regions account for only about 36.6 million of the 77.3 million total cycles. Roughly 40.7 million cycles remain in other work and have not yet been separately accounted for.

## Framebuffer internals

Framebuffer processing costs about 11.2 million cycles (~187 ms), but its individual expensive operations have not been isolated.

## BG FIFO internals

BG FIFO costs about 5.3 million cycles (~88 ms), but its individual expensive operations have not been isolated.

## Why ppu_drawing_mode() runs 24,198 times

The count is known, but the complete execution/call relationship producing that count has not yet been traced.

## Sprite FIFO behavior

Sprite FIFO measured zero cycles in this particular test frame. This does not establish that Sprite FIFO is universally unused.

## Correctness after diagnostic changes

Removing sprite handling is a performance experiment only. It is not an acceptable final optimization because sprite rendering correctness has not been preserved by that diagnostic change.

# 7. Hypothesis status

| Question / hypothesis | Evidence | Status |
| --- | --- | --- |
| PPU is the main performance problem | PPU ON ~1194 ms/frame; PPU OFF ~173 ms/frame in controlled earlier test. | CONFIRMED |
| Sprite Scan is a major bottleneck | ~20.0M cycles / ~334 ms. | CONFIRMED |
| Sprite-check loop causes Sprite Scan cost | Removing loop: ~1285 → ~940 ms; Sprite Scan → 0. | CONFIRMED |
| get_sp_byte() is the main cause | Replacing it with dummy value did not reduce Sprite Scan. | NOT SUPPORTED |
| The loop's exact expensive operation is known | No individual operation has yet been isolated. | NOT EXPLAINED |
| All frame cost has been accounted for | Measured regions total ~36.6M vs ~77.3M total. | NOT EXPLAINED |
| Sprite FIFO is a current bottleneck | Measured 0 cycles in tested frame. | NOT OBSERVED |

# 8. Investigation sequence so far

PPU enabled: roughly 1.19 s/frame.

PPU disabled: roughly 173 ms/frame.

Targeted PPU experiments established that PPU work, rather than the basic CPU/core path, dominates the frame.

DWT profiling was introduced to measure individual PPU sections.

Sprite Scan measured roughly 20M cycles.

24,198 drawing-mode calls were measured in one frame.

Removing the sprite-check loop reduced frame time by roughly 345 ms.

Replacing get_sp_byte() with a dummy value did not reduce the Sprite Scan cost.

# 9. Next controlled experiment

The next experiment should isolate the sprite loop itself. Temporarily replace the sprite-check body with a pure ten-iteration loop that performs no MMU/OAM access, LCDC access, comparisons against emulator state, or state writes. Measure Sprite Scan again.

Example diagnostic loop:

for (u8 i = 0; i < 10; ++i)\n{\n    if (i == 255)\n    {\n        ppu.oam_data.consumed[i] = true;\n        ppu.fifo.sprite_fetch_index = i;\n        ppu.sprite_fifo_is_running = true;\n        break;\n    }\n}

This is a diagnostic-only build and may disable sprite behavior. The purpose is to establish the baseline cost of the loop/control flow itself. After that, operations should be added back one at a time and measured.

# 10. Optimization rules for this investigation

Change one thing at a time.

Measure one frame after each controlled change.

Do not optimize based only on intuition when a measurement can isolate the cause.

Keep the Game Boy core independent of STM32-specific DWT hardware.

Treat diagnostic removals as temporary experiments, not final fixes.

After identifying the bottleneck, verify that the optimization preserves emulator behavior and framebuffer output.

# 11. Bottom line

The broad problem is now proven: the PPU dominates frame time. A major specific bottleneck is also proven: the sprite-check section of ppu_drawing_mode() costs about 20M cycles and removing it saves about 345 ms/frame. What remains unknown is exactly why that loop costs so much, and where the other roughly 40M cycles of the frame are spent. The correct next step is micro-level isolation of the sprite loop, followed by profiling the unaccounted total-cycle region.